Operation Manual

of

Frequency Writing Software

of

Integrated Equipment

| File Status [√] Draft [ ] Being revised [ ] Formally released | File Number: |  |
| --- | --- | --- |
|  | Version Number: | 1.0 |
|  | Project Name: | Frequency Writing Software of Integrated Equipment |
|  | Writer: |  |
|  | File Name: | Operation Manual of Frequency Writing Software of Integrated Equipment.doc |
|  | Secret Level: | Open |
|  | Verifier/ Department: |  |
|  | Completion Date: |  |

| Version | Date | Revision | Content | Written by |
| --- | --- | --- | --- | --- |
| 1.0 | 2014-11-05 | Full text | 1st Version |  |

Contents

1 Introduction	1

1.1 Executive Summary	1

1.2 Operation Environment	1

1.3 Software Interface	1

1.4 Software Modes	2

2 Installation	2

3 Frequency Writing Procedure	3

3.1 Preparation	3

3.2 Procedure	3

4 Functional Modules	4

4.1 Toolbar Buttons	4

4.2 Working Mode	6

4.3 Channel IP	10

4.4 Channel Frequency	11

4.5 Fixed Frequency	13

4.6 Power Supply Information	14

4.7 Power Information	15

4.8 Version Information	17

- 1 Introduction

- 1.1 Executive Summary

Frequency writing software of integrated equipment (hereinafter referred to as the "frequency writing software") is a software used for reading and setting of channel parameters of integrated equipment, such as ad-hoc networks, repeaters and ship-mounted radios.

- 1.2 Operation Environment

1.2.1 Hardware Environment

CPU: Dominant frequency 2.5GHz or above

Memory: 2G or above

Interface: USB 2.0

Frequency Writing Cable: HID frequency cable

1.2.2 Software Environment

Operating System: Window XP、VISTA、Window 7

Driver of frequency writing cable: insert the HID panel of the frequency writing cable into the host computer

Note: need to get the company’s authentication before use

- 1.3 Software Interface

The frequency writing software interface is as shown in Figure 1.

Figure 1 Software Interface

Meanings of Serial numbers in the diagram:

①：菜单栏，包括软件的功能选项列表以及帮助。

①=Menu bar: including functional options of the software and the ‘Help’.

②：工具栏，用于快捷地操作一些常用的功能，如打开设备、导入导出写频数据以及复位等操作。

②=Toolbar: used for fast operation of some commonly used functions, such as opening the equipment, import and export of frequency writing data and reset, etc.

③：目录树，用于选择需要操作的视图。

③=Directory tree: used to select views to operate.

④：操作视图。这是用户具体进行写频操作的界面部分。

④=Operation view: interface on which the user writes frequencies.

⑤：状态栏，用于显示软件当前运行的状态消息。

⑤=Status bar: used to display the current status of the software.

- 1.4 Software Modes

Software modes include ad hoc network, repeater and ship-mounted radio.

Click on "data - > mode" on the menu bar, a dialog box for software mode switching will pop out.

- 2 Installation

This frequency writing software is a green software. After getting our authorization of software, the user needs to copy or extract the software to a desired directory, and the installation is finished. Double-click on the main program to run the software.

Note: software running environment in some operating system may be incomplete, thus the software can not run. Under such circumstance, the user should extract "vcredist_x86.rar" in the installation directory, and run the "vcredist_x86.exe" to fix the system running environment.

- 3 Frequency Writing Procedure

- 3.1 Preparation

Prepare the frequency writing software, HID frequency writing cable and integrated equipment. Insert the USB end of the HID box of the frequency writing cable into the host computer, and insert the square port of the frequency writing cable into the CAN port of integrated equipment.

- 3.2 Procedure

The frequency writing procedure includes three steps: opening equipment, reading and writing frequency. As shown in Figure 2.

Figure 1 Frequency writing procedure

- 4 Functional Modules

- 4.1 Toolbar Buttons

The toolbar is shown in Figure 3, including opening equipment, import and export of data and reset.

Figure 2 Toolbar Diagram

4.1.1 Opening Equipment

The opening equipment button is as shown in Figure 4.

Figure 3 Opening Equipment

Click the button, it will pop open the equipment settings interface, as shown in Figure 5.

Figure 4 Opening the Equipment Settings Interface

Select "STM32" equipment in the "USB equipment list", choose corresponding operating rate and monitoring type (usually with default value) according to engineering requirements, and click "OK ".

When the user cannot find any "STM32" equipment in the "USB equipment list", and the equipment has been correctly connected with and identified by the hardware, click "Refresh" to update the equipment list, until "STM32" appears in the equipment list, then repeat the opening operation.

4.1.2 Data Importing

Data importing is as shown in Figure 6.

Figure 5 Data Importing Button

Import the preset frequency writing templates into the software via the data importing button, in order to simplify the complex operation of ticking options.

4.1.3 Data Exporting

Data exporting is as shown in Figure 7.

Figure 6 Data Exporting Button

Make frequency writing templates with preset data via the data exporting button, so as to simplify the frequency writing later on.

4.1.4 Totally Reset

The total reset button is as shown in Figure 8.

Figure 7 Total Reset Button

After the equipment is successfully opened, the user can take totally reset operation on integrated equipment through the totally reset button.

- 4.2 Working Mode

4.2.1 Local Channel

The parameter interface of ‘Local Channel’ under ‘Working Mode’ is as shown in Figure 9.

Figure 8 Working Mode - Parameter Interface of Local Channel

Specific meanings of working mode parameters is shown in Table 1.

Table 1 Description on Working Mode Parameters

| Name | Description |
| --- | --- |
| Wired link | A general option. Select it when the channel applies to wired calls. As for wireless channels, do not select this option. |
| Base station mode | A general option. Selected, the base station is in normal working mode; otherwise in radio working mode. |
| Dedicated control channel | A general option. Selected, the channel is the control channel. |
| PDT Mode | used for digital trunking. Selected, the channel applies to radios with PDT signaling verification. Otherwise, the channel applies to radios with DMR signaling verification. |
| Service suspending | A general option. Selected, the user can suspend the channel in remote control. |
| Radio login | The radio must send log in request before allowed to access the network. Otherwise, the radio can access the network without log in request. |
| Radio management | used for digital trunking. Selected, the authentication function is active. This function needs to be used for coordination with the authentication service or software authentication. |
| Radio information | Selected, the channel will upload the activity information of the radio, such as radio log in, log out, radio PTT status, etc. |
| Radio positioning | Selected, it will collect radio positioning information. |
| Ring mode | This option is dedicated to single calls placed by PDT digital radios. When selected, the control channel will require the recipient radio to ring, the subscriber can decide whether or not to answer. |
| Open group call | Select it to enable the radio to call an unregistered group, which is automatically set as a local group. |
| Simulcast channel | When simulcast calls are assigned to a called station, they can only be assigned on the audio channel whose option of simulcast channel has been selected. |
| Conventional base station | Select this option to support conventional radio conversation. |
| Routing channel | As an assistant control channel, it applies to digital/analog dual modes. In an analog system, the user needs to choose the dedicated analog channel. |
| Open the database | the radio database function on the functional board, to accelerated the process of single calls. |
| Dedicated analog channel | When the control channel is an analog channel, this option should be selected. |
| Preoccupied channel | used for engineering tests. |
| Signaling maintain | In analog trunking, the PTT signaling of the radio controls audio forwarding, otherwise it is controlled by the GATE signaling. |
| Local long Tx | This option is exclusively used for analog calls. If selected, when the channel assigns analog calls, the channel is still in a transmitting status even if no radio is talking. |
| Automatic roaming | used for digital trunking. Broadcasting the control channel information of the adjacent base station, helping the radio searching for the strongest signal. But the control channel parameters of the adjacent base station should be registered at the same time. |
| Disable restriction | If selected, the audio channel only accepts disable requests placed by the calling radio in the current conversation. |
| Call forwarding | When the base station receives an unregistered group call, it will forward the call signaling to the registered base station. Or forwarding GTC signaling from other base stations to notify that a roaming radio accessing the talking channel. |
| IP networking | Multi-channel networking is achieved through the IP port of the CCM, mainly used in mini base stations. |
| Digital/Analog demonstration | used for engineering tests. |
| Analog audio 2.4K | used for engineering tests. |
| Dynamic frequency | If selected, it will assign frequency dynamically from the frequency database, otherwise it will assign fixed frequency. |
| Channel circulating | used for engineering tests. |

4.2.2 Link Channel

Parameter interface of Link channel under working mode is as shown in Figure 10. See description on working mode parameters in Table 1 above.

Figure 9 Working Mode -Link Channel Parameters

- 4.3 Channel IP

4.3.1 Near End IP

The parameter interface of ‘Near End IP’ under ‘Channel IP’ is as shown in Figure 11.

Figure 10 Channel IP- Near End IP Parameters

The near end IP is used for reading and setting network parameters of channel, IP networking between channels, or directly using the address when managing

IP port. The relevant parameters are shown as follows.

“IP 地址”：信道机IP 地址

IP Address: IP address of transceiver

“子网掩码”：信道机子网掩码

Subnet Mask: subnet Mask of transceiver

“网关地址”：信道机网关地址

Gateway Address: gateway address of transceiver

“管理端口号”：默认端口5000

Management Port No.: default port- 5000

“会议端口号”：默认端口5001

Conference Port No.: default port-5001

“MAC 地址”：信道机MAC 地址，MAC 地址要求唯一。

MAC Address: MAC address of transceiver, sole and only

4.3.2 Far End IP

The parameter interface of ‘Far End IP’ under ‘Channel IP’ is as shown in Figure 12.

Figure 11 Channel IP- Far End IP Parameters

Without using of switch, transceivers can form a network. But the user needs to register the transceivers’ IP address of the far end base station in the Far End IP registration interface.

- 4.4 Channel Frequency

4.4.1 Local Channel

Local Channel parameter under ‘Channel Frequency’ is as shown in Figure 13.

Figure 12 Channel Frequency-Local Channel Parameters

It is used for reading and setting of channel frequency, color code and system code of the local channel. Related instructions are shown as follows.

“信道频点”：本地信道频点。

Channel frequency: local channel frequency

“色码”：数字系统下使用的标识码，设置范围为0-15。

Color code: identification code used in a digital system, ranging 0-15.

“系统码”：数字系统下，系统码由网络类型、基站位置识别码和PAR组成的四位16进制的编码。

System code: under a digital system, system code is a 4 bit 16 hexadecimal code, comprising of network type, ID code of base station location and PAR.

4.4.2 Link Channel

Link channel parameter under ‘Channel Frequency’ is as shown in Figure 14.

Figure 13 Channel Frequency-Link Channel Parameters

It is used for reading and setting of channel frequency, color code and system code of the Link channel. Related instructions are shown as follows.

“信道频点”：本地信道频点。

Channel frequency: link channel frequency

“色码”：数字系统下使用的标识码，设置范围为0-15。

Color code: identification code used in a digital system, ranging 0-15.

“系统码”：数字系统下，系统码由网络类型、基站位置识别码和PAR组成的四位16进制的编码。

System code: under a digital system, system code is a 4 bit 16 hexadecimal code, comprising of network type, ID code of base station location and PAR.

- 4.5 Fixed Frequency

4.5.1 Local Channel

Local channel parameter under ‘Fixed Frequency’ is as shown in Figure 15.

Figure 14 Fixed Frequency-Local Channel Parameter

It is used for reading and setting of local channel frequency and subsonic parameters.

其中，

Among them,

发射频率：读取和设置当前模块发射频率。

Tx frequency: read and set Tx frequency of the current module.

接收频率：读取和设置当前模块接收频率。

Rx frequency: read and set Rx frequency of the current module.

至于亚音参数设置，则需咨询我司工程人员。

As for settings of the subsonic parameters, the user should consult our engineering staff.

4.5.2 Link Channel

Link channel parameter under ‘Fixed Frequency’ is as shown in Figure 16.

Figure 15 Fixed Frequency-Link Channel Parameter

It is used for reading and setting of link channel frequency and subsonic parameters.

其中，

Among them,

发射频率：读取和设置当前模块发射频率。

Tx frequency: read and set Tx frequency of the current module.

接收频率：读取和设置当前模块接收频率。

Rx frequency: read and set Rx frequency of the current module.

至于亚音参数设置，则需咨询我司工程人员。

As for settings of the subsonic parameters, the user should consult our engineering staff.

- 4.6 Power Supply Information

Power supply information parameter is as shown in Figure 17.

Figure 16 Power Supply Information

It is used for checking the status of power supply module, power supply voltage, current, temperature, temperature alarm, etc.

电源开关：首先点击“读取”按钮，确认当前信道的电源状态，点击“开启”打开电源，点击“关闭”将信道机电源关闭。

Power switch: click the "read" button, confirm the power status of the current channel, then click "open" to open the power supply, or click "close" to shut down power supply of transceiver.

电压：显示信道机电源电压值、电瓶电压值。

Voltage: displaying power supply voltage or battery voltage of transceiver.

电流：显示信道机电流值。

Current: displaying current value of transceiver.

温度：显示信道机电源温度，设置温度告警门限。当温度高于设置的告警门限值时，信道机会发送告警信息给网管终端。

Temperature: displaying power supply temperature of transceiver, setting temperature alarm threshold. When the temperature is higher than the alarm threshold, the transceiver will send alarm information to the network management terminal.

- 4.7 Power Information

It includes the left PA and the right PA. Related parameters as as shown in Figure 18 and Figure 19.

Figure 17 Power Information - the Left PA

Figure 18 Power Information - the Right PA

It is used for displaying the running state of power amplifier, setting the forward power and alarm threshold. Read the "Terminal Board" information at the bottom left corner before use.

（1）左功放

(1) The left PA

功率值：点击“读取”按钮，读取当前前向功率值和告警状态。选择前向功率值，点击“设置”按钮进行设置。

Power value: click the "read" button to read the current forward power and alarm status. Select the forward power value, click the "Set" button to make settings.

低功率告警：设置低功率告警门限值，当前向功率小于门限值时，信道就会发送低功率告警信息到网管终端。

Low power alarm: set the low power alarm threshold, when the forward power is less than the threshold, the transceiver will send low power alarm to the network management terminal.

(2) The right PA

功率值：点击“读取”按钮，读取当前前向功率值和告警状态。选择前向功率值，点击“设置”按钮进行设置。

Power value: click the "read" button to read the current reverse power and alarm status.

High H

hHighHpower alarm: set the high power alarm threshold, when the reverse power is greater than the threshold, the transceiver will send high power alarm to the network management terminal.

- Temperature

- 功率值：点击“读取”按钮，读取当前前向功率值和告警状态。选择前向功率值，点击“设置”按钮进行设置。

Temperature: click the "read" button to read the current temperature and alarm status.

High temperature tertalarm: set the high temperature alarm threshold, when the temperature is greater than the threshold, the transceiver will send high temperature alarm to the network management terminal.

- 4.8 Version Information

The Version Information interface is as shown in Figure 20.

Figure 19 Version Information Interface

Version information includes information on CCM, local long Tx, link transceiver and the terminal board version. Click the "read" button to display the relevant version information. Click the "reset" button to reset the equipment.